SETUP AFTER RAM CLEAR OR INSTALLING v.2.8

- FOR GAP AND CONTINUOUS STRIP

a) Press 1 then Mode key.

b) Press ↑ key twice to go to SET then press PRT key.

c)  Press 4  then press PRT ( LABEL ENTER 1) key then press 5 (CLABEL)

d) Press 0 for standard gap labels and 1 for continuous strip labels. ( CLABEL IS DEFAULT TO 5 AFTER UPDATE)

e)  Press PRT key then “ C “ key.

Note: Turn Off PEELL SENSOR when using continuous strip. (page 44)

- FOR ½ AND ¼ KEY

  - Press 1 then Mode key.

  - Press ↑ key once to go to INIT mode.

  - Press PRT then enter the FF:

  - Press 0911 for disable or 0704 for enable.

e)   Press PRT key then “ C “ key.

- FOR SCALE TO SCALE TRANSFER

  - Attached Cable on both sending and receiving scale.

  - Press 2  then Mode key. (Receiving Scale)

  - Press ↑ once (TRANS) press the PRT key.

- Disable PASSWORD

  - Go to CAL Mode

  - Press 4 key then PRT key then 6 key.

  - Press 0 to disable and 1 to enable.

  - Press “0” key.

5.   Set up desired Label Format (DEFAULT TO LST8010 LABEL)