- PURPOSE

This document provides standard work instructions for an AV6010 read application with two or more AV6010’s.  It deals with setup and calibration of an AV6010 read point using two or more AV6010 cameras.

- SCOPE

- This document pertains to field installation of an AV6010 top read camera.

- RESPONSIBILITY

Primary:	Field Service

Support:	Tech Support, Application Engineering, Engineering and Test Engineering

- Prerequisites

Prior to installation a technician should be familiar with the basic functionality of a camera scanner.

- REFERENCES

Mounting drawings, app specifications, system overview

- REQUIREMENTS

| Tools Required |  |  |  |
| --- | --- | --- | --- |
| Lap Top PC | StaticFocusTarget.pdf | Net Connection | Install Kit |
| Tape Measure | CameraScanLineTarget.pdf | Mounting Drawing |  |
| Ethernet Cable | Camera mirror angle tool | Application Specs |  |
| Mirror Alignment tool |  | Image Viewer setup process rev 5.ppt |  |
| RangeFinder Calibration Rev 2.pdf |  |  |  |

| Software Required |  |
| --- | --- |
| Real Time Image Viewer | CodeWordViewer |
| RangerOScope | Install Wizard |

AV6010 Top Read Calibration

Inventory all equipment and structure parts.

- Review MOUNTING DRAWING and APPLICATION SPECIFICATIONS

  - Confirm that all equipment is mounted correct to the drawings and specifications

- RangeFinder

  - RangeFinder mounting

    - Centered over scanning area

      - If product is justified (left, right, or center) on a conveyor larger then 40 inches, then mount the RangeFinder to the justified side

      - Center the RangeFinder so the two laser dots are position equally distanced from the left and right side of the scanner area

Center LED lit indicates parallel with conveyor

Confirm  that the RangeFinder is parallel to the belt surface by only having the center LED lit in the status scale indicator on  the interface panel

    - Moun

    - RangeFinder beam blockers

      - Position the application tallest box on the conveyor to the far right or left of the scanning area. Adjust the beam blockers so that the RangeFinder scan line is still visible beyond the top surface of the box.

- AV6010 Mirror Alignment

  - Use the following alignment tool depending on the mounting angle requirement

    - 8 degrees 0-111550-001

    - 15 degrees 0-111551-001

    - 45 degrees 0-111549-001

- Trigger Photo Eye. (if applicable)

  - Confirm PE is square to the conveyor

  - Confirm PE is high enough off the conveyor surface that it will not get false triggers from any part of the conveyor

  - Confirm PE is aligned properly to the reflector.  (In a sender receiver application, make sure the two components are aligned correctly).

  - Confirm PE is 6 inches or greater from the RangeFinder scan line

The PE should be mounted according to the mounting drawing

- Connect PC to the sync network

  - Set your PC to 192.168.0.1

  - Confirm connectivity

    - IPconfig

      - Confirm your PC’s IP address

      - Show’s that your PC has an Ethernet connection to the AV6010

    - Ping

      - 192.168.0.50 (Camera)

      - 192.168.0.40 (RangeFinder)

  - Run Web Browser

    - Enter cameras IP into address line

      - 192.168.0.50

- Menu Setup

  - System Configuration

    - Camera Location

      - This setting is dependent on the position of the cameras in  the netowork

  - Setup

    - Tachometer

      - 20 or 16 tachs per inch

    - Focus and Detection

      - Package dimensions

      - Focus

        - RangeFinder, Fixed Focus, Light Curtain, etc.

      - Package Detection

        - Photo Eye, RangeFinder, Light Curtain, etc.

  - Symbologies

    - Set the bar code symbologies to the customers specs found in the application specifications.

  - Communication

      - Transmit point

        - Confirm with customer:

          - Transmit point

          - Box edge

      - Serial Comm (If applicable)

        - Enter customer specs

          - Protocol

      - Host Network (If applicable)

        - Enter customer specs

          - Protocol

      - Image Network (if applicable)

        - Top:

          - 10.0.40.21

      - Note:

        - Set up host message to accommodate special code condition such as a function character in a code 128 bar code.

- Static Setup

  - Connect to Sync Network

  - Run AV6010 Install Wizard for each component in the tunnel

    - Install Wizard software is found on the AV6010 web page under: Diagnostics>Tools>InstallWizard.zip

    - Use version InstallWizard_1.2.7.1 or greater

    - Use alignment targets

      - StaticFocusTarget.pdf

      - CameraScanLineTarget.pdf

  - Calibrate the RangeFinder

    - Level RangFinder mount

    - Setup according to document:

      - RangeFinder Setup 5_01_09 .pdf

- Dynamic  Setup

  - Change your IP to 10.0.40.19

  - Run the AV6010_RealTime_Viewer.exe application software

    - Install Wizard software is found on the AV6010 web page under: Diagnostics>Tools>ImageViewer.zip

  - Run conveyor at application speed

  - Refer to document:

    - Image Viewer setup process rev xx.ppt

- Confirm communication with host

  - Connect your PC to the host port

    - Serial

      - Run HyperTerminal

    - Ethernet

      - Net Connection

- Read Test

      - Test Boxes

      - Customer small box:

- Used to test the far read distance from the camera

      - Customer medium box:

- Used to test the center of the read area

      - Customer tall box:

- Used to test the near read distance from the camera

    - Code Placement

      - Obtain customer codes samples

        - Various density

        - Various contrast

      - Position the bar code on the surface of the box in the following manner (sample)

        - Note:  For a front or back read the top surface of the box should have labels placed in the same pattern to accommodate testing both surfaces at the same time

Front Left

Back Left

Back Right

Front Right

  - This step should be repeated for each AV6010 at the scan point in the tunnel

- Place bar codes on the surface of the box facing the AV6010 mounting location

- Use the small box with the bar codes on the top surface

- Run the box on the left, center, and right of the Field of View.

- All bar codes should read.

- Use the medium box with the bar codes on the top surface

- Run the box on the left, center, and right of the Field of View.

- All bar codes should read.

- Use the larger box with the bar codes on the top surface

- Run the box on the left, center, and right of the Field of View.

- All bar codes should read.

    - If there is a problem with reading of the bar codes, then run the Image Viewer setup process rev 2.pps on the Top AV6010.  These code should reflect a sampling of the various codes to be read by the scanner

      - Variation in contrast

      - Variation in codes X dimensions

    - Confirm that all codes read

- Observe sort

  - Confirm that all cameras are reading the bar code correctly and that the scan point is communicating to the host.