AV6010 Top Read First-Time Calibration Checklist

Confirmed that equipment is mounted according to the mounting dwg and app specs

Confirm that all equipment electrically installed properly.

Align the AV6010 mirror

Unlock camera head

Confirm PE is mounted and aligned correctly

Adjust RangeFinder beam blocker

Establish a connection and login to the system.

Menu Settings

System Configuration

Assign system name and positions of each camera in the system.

Tach Configuration

Focus and Package Info

Symbologies

Communication

Confirm transmit point and box edge with customer

Host network/Serial communications

Set special bar code comm. configuration (if necessary)

Image network (VisionCapture/IDS, optional)

Static Calibration

Wizard

Level RangeFinder

Calibrate RangeFinder (RangeFinder Calibration Rev 2 .doc)

Image Viewer

Dynamic Focus Calibration (AV6010_RealTime_Viewer.exe)

Run read test

Confirm communication with host and/or PLC

Watched sort

Save XML file to engineering folder

| Customer name: |  | Date: |  |
| --- | --- | --- | --- |
| Order Number: |  | Technician: |  |