- PURPOSE

This document provides standard work instructions for an AV6010 top read application.  It deals with setup and calibration of an AV6010 single read camera.

- SCOPE

- This document pertains to field installation of an AV6010 top read camera.

- RESPONSIBILITY

Primary:	Field Service

Support:	Tech Support, Application Engineering, Engineering and Test Engineering

- Prerequisites

Prior to installation a technician should be familiar with the basic functionality of a camera scanner.

- REFERENCES

Mounting drawings, app specifications, system overview

- REQUIREMENTS

- Tools Required:

| Lap Top PC | StaticFocusTarget.pdf | Net Connection | Install Kit |
| --- | --- | --- | --- |
| Tape Measure | CameraScanLineTarget.pdf | Mounting Drawing |  |
| Ethernet Cable | Camera mirror angle tool | Application Specs |  |
| Mirror Alignment tool |  |  |  |
| RangeFinder Calibration Rev 2.pdf |  | Image Viewer setup process rev 5.ppt |  |

AV6010 Top Read Setup

Inventory all equipment and structure parts.

Build structure and mount product according to the drawings.

- Review MOUNTING DRAWING and APPLICATION SPECIFICATIONS

  - Confirm that all equipment is mounted correct to the drawings and specifications

- Trigger Photo Eye. (if applicable)

  - Confirm PE is square to the conveyor

  - Confirm PE is high enough off the conveyor surface that it will not get false triggers from any part of the conveyor

  - Confirm PE is aligned properly to the reflector.  (In a sender receiver application, make sure the two components are aligned correctly).

  - Confirm PE is 6 inches or greater from the RangeFinder scan line

    - The PE should be mounted according to the mounting drawing

- RangeFinder

  - RangeFinder mounting

    - Centered over scanning area

      - Product may be justified (left, right, or center)

    - Mount RangeFinder beam blockers

      - Adjust using the applications tallest box

- AV6010 Mirror Alignment

  - Use the following alignment tool depending on the mounting angle requirement

    - 8 degrees 0-111550-001

    - 15 degrees 0-111551-001

    - 45 degrees 0-111549-001

Connect PC to the sync network

  - Set your PC to 192.168.0.1

    - Note

      - When the conveyor is running use the custom Ethernet cable for connecting a PC to the SYNC network. (PN:  0112247002)

  - Confirm connectivity

    - IPconfig

      - Confirm your PC’s IP address

      - Identifies PC Ethernet connection to the AV6010

    - Ping

      - 192.168.0.50 (Camera)

      - 192.168.0.40 (RangeFinder)

  - Run Web Browser

    - Enter cameras IP into address line

      - 192.168.0.50

- Menu Setup

  - Login in

    - User Id

      - Setup

    - Password

      - Setup

  - System Configuration

    - Camera Position

      - Select:

        - Top

  - Setup

    - Tachometer

      - Internal

      - External

        - 20 or 16 tachs per inch

    - Focus and Detection

      - Confirm the following information from the application specifications and confirmed with customer

      - Package dimensions

      - Focus

        - RangeFinder, Fixed Focus, Light Curtain, etc.

      - Package Detection

        - Photo Eye, RangeFinder, Light Curtain, etc.

  - Symbologies

    - Set the bar code symbologies to the customers specs found in the application specifications.

  - Communication

    - Top

    - Confirm the following information from the application specifications and confirmed with customer

      - Transmit point

        - Confirm with customer:

          - Transmit point

          - Box edge

      - Serial Comm (If applicable)

        - Enter customer specs

          - Protocol

      - Host Network (If applicable)

        - Enter customer specs

          - Protocol

      - Image Network (if applicable)

        - Top:

          - Set AV6010 image static IP address

            - 10.0.40.21

      - Note:

        - Set up host message to accommodate special code condition such as a function character in a code 128 bar code.

- Static Setup

  - Connect to Sync Network

  - Run AV6010 Install Wizard for each component in the tunnel

    - Install Wizard software is found on the AV6010 web page under: Diagnostics>Tools>Click here for Utilities and Documentation>InstallWizard.zip

    - Use version InstallWizard_1.2.7.1 or greater

    - Follow all the steps in the Install Wizard

      - Use alignment targets

        - StaticFocusTarget.pdf

        - CameraScanLineTarget.pdf

  - Calibrate the RangeFinder

    - Level RangeFinder mount

    - Setup according to document:

      - RangeFinder Calibration Rev 2 .pdf

- Dynamic  Setup

  - Change the Ethernet cable to the Image port of the AV6010

  - Change your IP to 10.0.40.19

  - Run the AV6010_RealTime_Viewer.exe application software

    - Install Wizard software is found on the AV6010 web page under: Diagnostics>Tools>ImageViewer.zip

  - In the AV6010 Real Time Viewer select the 10.0.40.21 as the Cam IP

  - Run conveyor at application speed

  - For step by step process refer to document:

    - Image Viewer setup process rev 5.ppt

- Confirm communication with host

  - Connect your PC to the host port

    - Serial

      - Run HyperTerminal to capture host messages sent from the AV6010

    - Ethernet

      - Run Net Connection to capture host messages sent from the AV6010

- Read Test

      - Test Boxes

      - Customer small box:

- Used to test the far read distance from the camera

      - Customer medium box:

- Used to test the center of the read area

      - Customer tall box:

- Used to test the near read distance from the camera

    - Code Placement

      - Obtain customer codes samples

        - Various densities

        - Various contrasts

      - Position the bar code on the surface of the box in the following manner (sample)

Front Left

Back Left

Back Right

Front Right

  - Top AV6010

- Place customer bar codes on the top surface of the box

      - Use minimum and maximum x dimension bar code on all sizes boxes

- Use the small box with the bar codes on the top surface

- Run the box on the left, center, and right of the Field of View.

- All bar codes should read.

- Use the medium box with the bar codes on the top surface

- Run the box on the left, center, and right of the Field of View.

- All bar codes should read.

- Use the tall box with the bar codes on the top surface

- Run the box on the left, center, and right of the Field of View.

- All bar codes should read.

    - If there is a problem with reading of the bar codes, then run the Image Viewer setup process rev 2.pps on the Top AV6010.  These code should reflect a sampling of the various codes to be read by the scanner

      - Variation in contrast

      - Variation in codes X dimensions

    - Confirm that all codes read

- Observe sort

  - Confirm AV6010 camera is reading bar codes correctly

  - Confirm that the boxes are sorting correctly

- Save

  - XML

  - Submit XML files as an attachment to the daily installation update email.

    - Copy the application engineer