DIGI

MI-2600E

Touch 1 next to internal count

Adjust the ZERO ORIGINAL count   with dip switch SW2 on the A/D  board to as near to 5000 as possible. This count must be set to  between 3000 and 7000, if this is not possible there is a fault in the weighing circuit.

Touch RESET to zero COUNT AFTER SPAN

Place the indicated weight (e.g. 4Kg) on the scale and touch EXEC.

When AUTO SPAN COMPLETE is displayed, remove the weight from the scale, ZERO AFTER SPAN should read 5000, when it does touch RESET to zero COUNT AFTER SPAN

When calibration is complete touch EXIT

The steps 4 and 5 should be repeated several times.

FIN