Codes d'erreur

| oLoAd | Oload indicates that an overload conditions exists, that is, the gross weight as measured from the zero value entered at calibration has exceeded 102.5% of the scale capacity. |
| --- | --- |
| ULoAd | Uload indicates that an underload conditions exists, that is, the gross weight as measured from the zero value entered at calibration is less than 2% of the scale capacity. |
| noPrnt | This indicates that printing was inhibited by an overload, underload, or scale motion condition. |
| bUSY | This indicates that printing was inhibited by the printer's busy signal being active. |
| oFlo | This indicates that the number to be displayed in the current mode is greater than 999999 disregarding any decimal point. |
| UFlo | This indicates that the number to be displayed in the current mode is less than -999999 disregarding any decimal point. |
| no Add | This indicates that even though an accumulator key was pressed, no accumulation took place. This is because an overload, underload, or scale motion condition existed, OR accumulation has already taken place on PRINT or ENTER and the weight on the scale has not changed plus or minus by at least the minimum accumulate weight from the last weight accumulated, OR the indicator was powered-up with weight displayed and an accumulation was attempted before the weight is reduced to less than the minimum accumulate weight. |