CODES D'ERREUR

| Error messages shown | # Error status |
| --- | --- |
| Err | Weighted value is over the internal designated upper limit, the offset value is too high, and/or the reference ZERO is too high. |
| -Err | Weighted value is below the internal designated lower limit and/or the offset value is lower than zero. |
| AdErr | Malfunction in A/D conversion and/or the AD IC. |
| Eerr | Malfunction in parameters memory storage and/or the EEPROM IC. |
| unSt | The scale is at unstable state. |
| ------- | The weight load is over the maximum indication, that is, Max, Capacity +9e. |