QI Personal Compilation

# Cyclic Message Format

Cyclic Message Format for Multiple APC Instrument Channels

The Q.iMPACT cluster sends real-time process data to the Controller through “Cyclic Input-to-Controller Assembly” messages that are 496 bytes long.  These are Class 1 scheduled messages.

An “Instrument Channel” is the logical representation of a physical measurement device, either a scale or flow meter.  There are up to 200 instrument channels in a Q.iMPACT cluster, numbered from 1 to 200.  A Controller issues commands to a physical device using its instrument channel number.  You assign the instrument channel numbers during system configuration.

Data from each instrument channel fits into an “Assembly Slot” within the Cyclic Input messages.  Cyclic input data from each instrument channel, or the size of each assembly slot, is 20 bytes long.  The format of the data block is the same for all instrument channels.  There are 24 assembly slots in one 496-byte Cyclic Input-to-Controller Assembly message with 16 bytes reserved for future growth.  Assembly slots are numbered 1 to 24 within each Assembly message.

Within a cluster, “Q.iMPACT Bridge” terminals contain a ControlNet interface card that communicates to a Controller.  Each bridge terminal has an Assembly message structure for up to 24 instrument channels.  It also contains a message router that sends command messages to remote Q.iMPACT terminals.

There is a fixed assignment of  “instrument channels” to “assembly slots” within each bridge terminal.  That is, each instrument channel has a fixed assembly slot within the Cyclic Message Assembly of a bridge terminal. Q.iMPACT assigns instrument channels to assembly slots sequentially within the bridge terminal, during system configuration.  The instrument channels may be local to the bridge terminal or they may be in remote Q.iMPACT terminals.  For most efficient operation of the cluster, you should configure as many instrument channels as possible to the local Assembly structure.  A Controller application must know the assembly slot for each instrument channel in order to get the real-time data for each device.

There must be a separate Q.iMPACT ControlNet Interface Card for every 24 instruments in the Q.iMPACT Cluster.  For example, to support a cluster with 96 instruments, you would need four ControlNet Interface cards in the cluster in four separate Q.iMPACT terminals.  A Q.iMPACT cluster can contain up to 200 instruments.

The following table shows the arrangement of assembly slots within the larger assembly message for multiple instrument channels.

| Reserved |  | Data Byte 1 | Data Byte 2 | ………… | Data Byte 16 |
| --- | --- | --- | --- | --- | --- |
| Assembly Slot 1 |  | Data Byte 17 | Data Byte 18 | ………… | Data Byte 36 |
| Assembly Slot 2 |  | Data Byte 37 | Data Byte 38 |  | Data Byte 56 |
| Assembly Slot 23 |  | Data Byte 457 | Data Byte 458 |  | Data Byte 476 |
| Assembly Slot 24 |  | Data Byte 477 | Data Byte 478 | …………… | Data Byte 496 |

The Q.iMPACT normally updates data for the Cyclic Input-to-Controller Assembly once a second.   In case of critical events, such as, the completion of a material movement operation or an error, the Q.iMPACT updates the status immediately.  The Controller sets the rate at which it cyclically reads the Assembly.

# Message #1. Q.iMPACT->Controller Cyclic Input Assembly for Single Instrument Channel

The Cyclic Input-to-Controller Assembly data for each instrument channel is 20 bytes long.  The table below shows the data format, which is the same for all instrument channels.

| Offset | “Data Descriptor Label” | Short Name | Data Type | Access | Internal Update Rate |
| --- | --- | --- | --- | --- | --- |
| 0 | Channel Number (1-200) | Channel | Byte | View Only | Once a Second |
| 1 | - “PAC Data Integrity Bit” alternates polarity every 5 seconds. - “Instrument Data Integrity OK “ - “Scale Over Capacity” - “Scale Under Zero” - “Scale Motion” - “Material Transfer Cycle Active” - “Final Control Element Output” 0 = Off , 1 = On - “Waiting for Controller to Acknowledge Last Material Transfer/Hand Add complete” | DataIntegrity DataOK OverCapacity UnderZero ScaleMotion CycleActive FCE_Output AwaitingACK | 8 Binary | View Only | Once a Second. Critical Events Immediately. |
| 2 | - “Feed Type” - “Feed Type” 0=Gain In Weight, 1= Loss In Weight, 2= Flow Meter, 3 = Hand Add - “Manual-Not Auto-Mode” - “Gross Weight Feed” - “Feed Override Active” – external logic inhibited from removing feed permissive - “Feed Failed” - “Communication Error” - “Device Stability Warning” - “Very Unstable Device” - “Too High or Too Low Flow” at cutoff - “Three Times Average Flow” at cutoff - “Fast Feed Rate Alarm” - “Wait for All Overlap Requests” - “Waiting to Start Primary” Overlapped Feed - “Primary Overlapped Feed” In Progress - “Secondary Overlapped Feed” In Progress | FeedType ManualMode GrossWeight FeedOverride FeedFailed CommError WgtUnstable VeryUnstable ErraticFlow 3TimesFlow RateAlarm WaitOvlpReq DelayPrimary PrimOverlap SecOverlap | 16 Binary | View Only | Once a Second during Material Transfers Critical Events Immediately. |
| 4 | “Feed Weight”. This field is reset to zero at the beginning of a feed. During most feeds, this field contains the Net Accumulated Weight for the single feed. During Primary Overlapped feeds, this field contains the combined weight of all feeds. At completion of the feed, it contains the Delivered Weight for this feed. | FeedWeight | IEEE Float 32 | View Only | Once a Second during Material Transfer Cycle. |
| 8 | “Gross Weight”. For scales, this field the gross weight. For flow meters, this field is the same as Feed Weight. | GrossWeight | IEEE Float 32 | View Only | Once a Second for Scales |
| 12 | “Rate of Change of Weight” | Rate | IEEE Float 32 | View Only | Once a Second during Material Transfers for Flow Meters and Scales. |
| 16 | “Time until Slow Step Timer Expires” in Seconds. 0 = Alarm. | SlowStepTimr | Integer | View Only | Once a Second during Material Transfers |
| 18 | “Estimated Time to Complete” in Seconds | TimeToFinish | Integer | View Only | Once a Second during Material Transfers |
|  | End of Message. Length = 20 bytes |  |  |  |  |

# Message #2. Controller->Q.iMPACT Command for Single Instrument Channel

A Controller sends a Shared Data Request Message to the Q.iMPACT to give a specific command to a specific instrument channel.  To send the message, the Controller Ladder program executes a MESSAGE instruction to “Set All Attributes in Class 84 (hex) Instance 1” which sets the APC Command “ACM00” Object in the Q.iMPACT.

The message table below shows the APC Command Object format.  The first four fields of this message are identical in this request message and in its matching response message.

| Offset | “Data Descriptor Label” | Short Name | Data Type | Access |
| --- | --- | --- | --- | --- |
| 0 | “Channel Number” ( 1 – 200 ) | Channel | Byte | Engineer |
| 1 | “Message Sequence Number”. The controller must generate a new sequence number for each new command. | SequenceNum | Byte | Engineer |
| 2 | “Material Path Index” ( 1- 1000 ) | MaterialPath | Integer | Engineer |
| 4 | “Command” Number - Start Material Transfer. - Start Material Transfer with Gross Weight Target. This command is only valid for a scale device. - Start Hand Add. - Acknowledge Material Transfer or Hand Add Complete - Abort Material Transfer - Reset Slow Step Timer - Start Manual Mode - Turn on FCE in Manual Mode - Turn off FCE in Manual Mode - Restart Auto Mode - Complete Feed in Manual Mode - Master Reset – Instrument channel - Report Last Status - Master Reset – Cluster - Validate Aggregate Secondary Feeds 30 Reset Local ControlNet Card 31 Reset all ControlNet Cards in Cluster | Command | Byte | Operator |
| The following fields are applicable only the “Start Material Transfer Cycle” Command |  |  |  |  |
| 5 | “Group Number” for Start Material Transfer Command. This field identifies which Primary and Secondary feed requests belong to the group of feeds that make up an overlapping feed. A value = 0 indicates that this NOT an overlapped feed request. | GroupNumber | Byte | Operator |
| 6 | “Number of Overlapping Secondary Feeds” that are being fed into a Unit simultaneously with THIS Primary Feed. This field is only meaningful in a Primary Feed using scale instrument for the Vessel. The Material-Path must indicate this Gain-In-Weight feed. The APC turns on the FCE for the Primary Feed when it determines there will be enough time after the overlap completes to run the APC algorithm with the scale. | OverlapNum | Byte | Operator |
| 7 | “Reserved” | Reserved | Byte | Engineer |
| 8 | “Target Weight” For “Start Material Transfer” or “Start Hand Add” commands, this field must contain the target weight for the feed. For “Turn On FCE In Manual Mode”, this field must contain the number of seconds to keep the FCE on”. The value “-9999” is a 32-year timeout. All other negative values are a 0-second timeout. For “Reset Slow Step Timer”, this field may contain the number of seconds to set the slow step timer. If this field is 0, the Q.iMPACT resets the slow step timer to its original value for the feed. | TargetWeight | IEEE Float 32 | Operator |
| 12 | “Tolerance +” Value “-9999” disables tolerance check. | PosTolerance | IEEE Float 32 | Operator |
| 16 | “Tolerance –“ Value “-9999” disables tolerance check. | NegTolerance | IEEE Float 32 | Operator |
| 20 | “Material Transfer ID” is an identifier field that is sent from the Material Transfer Controller. The Q.iMPACT reports this field as part of the data collection record. If there is a “~” in the field, the Q.iMPACT displays the data following the “~”. | MatTranID | 40 Character | Engineer |
|  | End of Message. Length = 60. |  |  |  |

# Message #3. Q.iMPACT->Controller Response for Single Instrument Channel

The following table shows the format of the APC Command Status Object, which is sent from the Q.iMPACT to the Controller.  To get the Command Status Object, the Controller Ladder program must execute a MESSAGE instruction to “Get All Attributes from Class 85 (hex) Instance 1” which gets the APC Command Status “ACS00” Object in the Q.iMPACT.  The first four fields of the Command Status Object are identical to the Command Object in Request Message #2.  If these fields do not match in the two messages, the Controller will know that the response is not valid and must take corrective action.

| Offset | “Data Descriptor Label” | Short Name | Data Type | Access |
| --- | --- | --- | --- | --- |
| 0 | “Channel Number” as in Message #2 | Channel | Byte | View Only |
| 1 | “Message Sequence Number” as in Message #2 | SequenceNum | Byte | View Only |
| 2 | “Material Path Index” as in Message #2 | MaterialPath | Integer | View Only |
| 4 | “Command” Number as in Message #2 | Command | Byte | View Only |
| 5 | “Command Status” See Listing below. | ComandStatus | Byte | View Only |
| The following fields typically are only applicable in the “Acknowledge Material Transfer Complete” Command Response. However, when a Start Material Transfer command fails immediately, these fields will contain values indicating a Material Transfer failure. |  |  |  |  |
| 6 | “Material Transfer Status” See Listing below. | MxferStatus | Byte | View Only |
| 7 | “Reserved” | Reserved | Reserved | View Only |
| 8 | “Material Transfer Status Qualifiers” - “Over Tolerance” - “Under Tolerance” - “Power Failure” during feed 3-15 Reserved | MxferStatQ | 16 Binary | View Only |
| 10 | “Reserved” | Reserved | Integer | View Only |
| 12 | “Delivered Weight” | FeedWeight | IEEE Float 32 | View Only |
| 16 | “Deviation Error from Target Weight” | TargetError | IEEE Float 32 | View Only |
|  | End of Message. Length = 20 |  |  |  |

# Command Status Values: (ARN05)

- SUCCESS – Start Gain-In-Weight Material Transfer Command Complete

- SUCCESS – Start Loss-In-Weight Material Transfer Complete.

- SUCCESS – Start Flow Meter Material Transfer Complete.

- SUCCESS – Start Validate Aggregate Feed Complete.

- SUCCESS – Start Hand Add Command Complete.

- SUCCESS –  Command Complete

- Command Not Complete – Request status again after a short delay

- ERROR – Communications Error

- ERROR – Invalid Instrument Channel Number

- ERROR – Invalid Command

- ERROR – Invalid Material-Path Table Index Number

- ERROR – Invalid Algorithm in Material-Path Table Entry

- ERROR – Invalid Feed Type in Material-Path Table Entry

- ERROR – Invalid Measuring Device Channel Table Index  in  Material Path Table Entry

- ERROR – Invalid Gain In Weight Feed and Dump to Empty Algorithm  Combination in Material Path Table

- ERROR – Invalid Destination in Material Path Table Entry.

- ERROR – Other invalid data in Material Path Table Entry

- ERROR – Overlap Feed Request Error, including invalid Loss In Weight Feed in Material Path Entry and Overlapping Feed Command.

- ERROR – Invalid data In Measuring Device Channel Table Entry

- ERROR – Invalid Mode for Command, e.g., Controller is requesting to start a new material transfer before the last feed is complete or before the controller has acknowledged that the last material transfer is complete.

- ERROR – Requested add amount too small

- ERROR – Requested add amount would bring Scale Device over capacity

- ERROR – Scale Device Currently over Capacity

- ERROR – Scale Device Currently under Zero

- ERROR – Instrument Malfunction

- ERROR – Target Weight is less than Spill

- ERROR – Response Timeout

- ERROR – Too many overlapping feeds

- WARNING – Delayed start to feed due to overlapping feed.

- WARNING – Abort ignored since Time to Complete was less than Feed Override Time

- ERROR – Invalid overlap group number

- WARNING – Waiting for All Secondary Requests.

- WARNING – Waiting for Measuring Device Stability.

- ERROR – Not Enough Material.

- ERROR – Device not configured or calibrated properly.

35  Warning – Secondary Feed Start queued.

36  Error – License violation, use Spill only

95  Error – Scale capacity problem

96  Error – Scale or Flowmeter problem

97  Warning – Scale Loss in Weight busy

98  Error – Aborted due to very unstable Scale

99 Error – No Overlapping Feed command being processed

# Material Transfer Status Values:  (ARN06)

- Successful Material Transfer – K1, K2 parameters updated

- Successful Material Transfer – Spill Only

- Successful Material Transfer- Dump to Empty

- Successful Hand Add

- Material Transfer Complete -  Parameters NOT updated

- Material Transfer Complete – Parameters reset.

- Material Transfer Complete with Manual Operation

- Failed – Unstable Scale

- Failed – Overlapping Feed Error Corrupted Flow

- Failed – Erratic Flow Error

- Failed  - Low Flow Error

- Failed -  High Flow Rate Alarm Error

- Failed – Communication Error

- Failed – Instrument Error

- Failed – Scale Device Capacity Error

- Failed – Predictive Algorithm Error

- Failed – Material Transfer with Manual Operation

- Failed – Amount of material transferred did not match in source and destination.

- Failed – Controller Aborted Material Transfer

- Failed – Controller Reset Channel

- Failed – Controller Reset Cluster

- Failed -  Reserved

- Failed – Slow Step Timer Timeout

- Failed – Secondary Requests Timeout

- Failed – Power Failure During Feed

- Failed – Start Material Transfer Command Failed Immediately – Transfer Did Not Start

- Status Only – Material Transfer Is In Progress.

(ARN07)

Transfer Status Qualifier Bits

Transfer Status Qualifier Bits. Set at completion of feed.

- Over Tolerance

- Under Tolerance

- Power Failure During Feed

# PAC Communications Command Data

ControlNet Class Code: 84 hex

- There is one instance of the PAC Command Data Object.

The Q.iMPACT stores these Shared Data fields in HEAP memory.  It only actively maintains these fields when it has a local Controller “Bridge” Interface.

A Controller sends a Shared Data Request Message to the Q.iMPACT to give a specific command to a specific instrument or Unit.  To send the message, the Controller Ladder program executes a MESSAGE instruction to “Set All Attributes” in the PAC Command “ACM00” Object in the Q.iMPACT.  The table below shows the PAC Command Object format.  The first four fields of this message are identical in this request message and in its matching response message.

- PAC Communications Command SHARED DATA

Attr. 	Function/Value			Name	Format	Description

- 1	Channel Number			acm01	UC	Instrument Channel for Command.

- 2	Message Sequence Number	acm02	UC	Message Sequence Number of Command.

- 3	Material Path Index		acm03	US	Material Path Index

4	Command Number		acm04	UC	“Command” Number.  Refer to “akn04”.

5	Overlap Feed Group		acm05	UC	Overlap Feed Group.  Refer to “akn05”.

6	Number of Overlapping Feeds	acm06	US	Number of Overlapping Feeds that are

simultaneously going into a Unit.  Refer to “akn06”.

7	Target Weight			acm07	F	Target Feed Weight.

8	Positive Tolerance		acm08	F	Positive Feed Tolerance.

9	Negative Tolerance		acm09	F	Negative Feed Tolerance.

10	Batch ID/ Display Message	acm10	40C	Batch ID from Host Controller that is used for

Data Collection Messages.   If there is a “^”

in the field, data following the “^” is used

as a Display Message for the Q.iMPACT

display.

# PAC Communications Command Status

ControlNet Class Code: 85 hex

- There is one instance of the PAC Command Status Object.

The Q.iMPACT stores these Shared Data fields in HEAP memory.  It only actively maintains these fields when it has a local Controller Interface.

The Q.iMPACT sends the PAC Command Status Shared Data to the Controller to give the immediate status for a command.  To get the Command Status Object, the Controller Ladder program must execute a MESSAGE instruction to “Get All Attributes” of the PAC Command Status “ACS00” Object in the Q.iMPACT.  The first four fields of the Command Status Object are identical to the Command Object in Request Message #2.  If these fields do not match in the two messages, the Controller will know that the response is not valid and must take corrective action.

- PAC Communications Command Status SHARED DATA

Attr. 	Function/Value			Name	Format	Description

- 1	Channel Number			acs01	UC	Instrument Channel for Command.

- Same as acm01.

- 2	Message Sequence Number	acs02	UC	Message Sequence Number of Command.

- Same as acm01.

- 3	Material Path Index		acs03	US	Material Path Index.  Same as acm01.

- 4	Command Number		acs04	UC	Command Number.  Same as acm01.

5	Command Status			acs05	UC	Command Status.  Refer to “arn05”.

6	Last Material Transfer Status	acs06	US	Last Material Transfer Status.  Refer to

“arn06”.

7	Transfer Status Qualifier Bits	acs07	L	Transfer Status Qualifier Bits.  Refer to

“arn07”.

8	Delivered Weight			acs08	F	Delivered Weight at Completion of Feed.

9	Deviation from Target Weight	acs09	F	Deviation from Target Weight

# Q.iMPACT->PLC Cyclic Communications Input Assembly

ControlNet Class Code: 86 hex

- There are 25 instances of the Q.iMPACT->PLC Input Assembly.  One instance is reserved for future growth.

The Q.iMPACT stores these Shared Data fields in HEAP memory.  It only actively maintains these fields when it has a local Controller Interface.

The Q.iMPACT sends the real-time process data to the Controller through the Q.iMPACT->Controller Cyclic Input Assembly message.  Each Q.iMPACT channel, representing either a weigh scale or flow meter, has a Q.iMPACT->Controller Input Assembly data block that is 20 bytes long.  The format of the block is the same for all channels.  The Q.iMPACT packs multiple instrument channels into one large Cyclic Input-to-Controller Assembly message.  24 instrument channels can fit snuggly into one 496-byte Class 1 message, with 16 bytes reserved for future growth.

This approach requires a separate Q.iMPACT Bridge terminal containing a ControlNet Interface Card for every 24 channels in the Q.iMPACT Cluster.  For example, to support a cluster with 96 channels, you would need four ControlNet Interface cards in the cluster in four separate Q.iMPACT terminals.  A Q.iMPACT cluster can contain up to 200 channels.

The Instrument Channel Configuration Object has the assignment of instrument channels to the Q.iMPACT Bridge terminal containing the ControlNet interface.  The following table shows the arrangement of instrument data within the larger assembly message for multiple instrument channels.

| Reserved |  | Data Byte 1 | Data Byte 2 | ………… | Data Byte 16 |
| --- | --- | --- | --- | --- | --- |
| Assembly Slot 1 |  | Data Byte 17 | Data Byte 18 | ………… | Data Byte 36 |
| Assembly Slot 2 |  | Data Byte 37 | Data Byte 38 |  | Data Byte 56 |
| Assembly Slot 23 |  | Data Byte 457 | Data Byte 458 |  | Data Byte 476 |
| Assembly Slot 24 |  | Data Byte 477 | Data Byte 478 | …………… | Data Byte 496 |

The Q.iMPACT normally updates data for the Cyclic Input-to-Controller Assembly once a second.   In case of critical events, such as, the completion of a material movement operation or an error, the Q.iMPACT updates the status immediately.  The Controller sets the rate at which it cyclically reads the Assembly.

The Cyclic Input-to-Controller Assembly data for each instrument channel is 20 bytes long.  Refer to the Process Table, which shows the format of the assembly.

# Q.iMPACT->Controller Cyclic Communications Input Assembly

Attr. 	Function/Value			Name	Format	Description

- 1	Reserved			aa101	16C	Reserved

- 2	Channel #1 PLC Input Assembly	aa102	20C	Channel #1 PLC Input Assembly

- 3	Channel #2 PLC Input Assembly	aa103	20C	Channel #2 PLC Input Assembly

- .	.	.			.	.	.

- .	.	.			.	.	.

- 25	Channel #24 PLC Input Assembly	aa125	20C	Channel #24 PLC Input Assembly

Q.iMPACT->PLC Cyclic Communications Input Assembly

ControlNet Class Code: 86 hex

- There are 25 instances of the Q.iMPACT->PLC Input Assembly.  One instance is reserved for future growth.

The Q.iMPACT stores these Shared Data fields in HEAP memory.  It only actively maintains these fields when it has a local Controller Interface.

The Q.iMPACT sends the real-time process data to the Controller through the Q.iMPACT->Controller Cyclic Input Assembly message.  Each Q.iMPACT channel, representing either a weigh scale or flow meter, has a Q.iMPACT->Controller Input Assembly data block that is 20 bytes long.  The format of the block is the same for all channels.  The Q.iMPACT packs multiple instrument channels into one large Cyclic Input-to-Controller Assembly message.  24 instrument channels can fit snuggly into one 496-byte Class 1 message, with 16 bytes reserved for future growth.

This approach requires a separate Q.iMPACT Bridge terminal containing a ControlNet Interface Card for every 24 channels in the Q.iMPACT Cluster.  For example, to support a cluster with 96 channels, you would need four ControlNet Interface cards in the cluster in four separate Q.iMPACT terminals.  A Q.iMPACT cluster can contain up to 200 channels.

The Instrument Channel Configuration Object has the assignment of instrument channels to the Q.iMPACT Bridge terminal containing the ControlNet interface.  The following table shows the arrangement of instrument data within the larger assembly message for multiple instrument channels.

| Reserved |  | Data Byte 1 | Data Byte 2 | ………… | Data Byte 16 |
| --- | --- | --- | --- | --- | --- |
| Assembly Slot 1 |  | Data Byte 17 | Data Byte 18 | ………… | Data Byte 36 |
| Assembly Slot 2 |  | Data Byte 37 | Data Byte 38 |  | Data Byte 56 |
| Assembly Slot 23 |  | Data Byte 457 | Data Byte 458 |  | Data Byte 476 |
| Assembly Slot 24 |  | Data Byte 477 | Data Byte 478 | …………… | Data Byte 496 |

The Q.iMPACT normally updates data for the Cyclic Input-to-Controller Assembly once a second.   In case of critical events, such as, the completion of a material movement operation or an error, the Q.iMPACT updates the status immediately.  The Controller sets the rate at which it cyclically reads the Assembly.

The Cyclic Input-to-Controller Assembly data for each instrument channel is 20 bytes long.  Refer to the Process Table, which shows the format of the assembly.

# Q.iMPACT->Controller Cyclic Communications Input Assembly

Attr. 	Function/Value			Name	Format	Description

- 1	Reserved			aa101	16C	Reserved

- 2	Channel #1 PLC Input Assembly	aa102	20C	Channel #1 PLC Input Assembly

- 3	Channel #2 PLC Input Assembly	aa103	20C	Channel #2 PLC Input Assembly

- .	.	.			.	.	.

- .	.	.			.	.	.

- 25	Channel #24 PLC Input Assembly	aa125	20C	Channel #24 PLC Input Assembly

## PAC Processes

There are 12 PAC Process Identifiers within a Q.iMPACT, and, at most, only 12 PAC Processes can be active at a time within each Q.iMPACT.  There can be 240 PAC Process Identifiers in a Q.iMPACT cluster, but there are can be at most 200 active PAC Processes in a Q.iMPACT cluster.  When there are one to four scales and up to eight flow meters in the Q.iMPACT, the assignment of devices to processes within a Q.iMPACT is as follows:

Process 1 	Flow Meter K		|	Process 2	Flow Meter L
Process 3 	Flow Meter M		|	Process 4 	Flow Meter N

Process 5 	Flow Meter O		|	Process 6 	Flow Meter P

Process 7	Flow Meter Q		|	Process 8	Flow Meter R
Process 9 	Scale A 		|	Process 10	Scale B

Process 11	Scale C 		|	Process 12	Scale D

When there are no scales and up to 12 flow meters in the Q.iMPACT, the assignment of Processes to Flow Meters is as follows:

Process 1 	Flow Meter K		|	Process 2	Flow Meter L
Process 3 	Flow Meter M		|	Process 4 	Flow Meter N

Process 5 	Flow Meter O		|	Process 6 	Flow Meter P

- Process 7	Flow Meter Q		|	Process 8	Flow Meter R
Process 9	Flow Meter S		|	Process 10	Flow Meter T
Process 11	Flow Meter U		|	Process 12	Flow Meter V

## Measuring Device Channel  Table

Each Measuring Device Channel Table entry contains several static parameters associated with flow control for each measuring device.  A Measuring Device is a Scale or Flow Meter.  The System Engineer initially sets up these parameters at system installation and only changes them to “fine tune” or to reconfigure the system.   Some parameters are only applicable to scale devices.

ControlNet Class Code: 8A hex

An instance of the Measuring Device Channel Table Object may exist for each of 200 instruments in a Q.iMPACT cluster.  Each contains the numbered instance attributes.

- MEASURING DEVICE CHANNEL TABLE SHARED DATA

- Where the Hex number  ‘nn’ is the instance representing the Measuring Device 1-200.

Attr. 	Function/Value			Name	Format	Description

1	Measuring Device Assigned	unn01	UC	Setup. 1=Yes, 0=No

2	Q.iMPACT Terminal		unn02	C	Setup.  0, 1-9, A-K.  Cluster Terminal

containing Measuring Device.

3	Measuring Device		unn03	C	Setup.  Scale A – D or Flow Meter K – V.

4	Stable Scale Time		unn04	US	Setup. Number of seconds to wait for a

stable scale  reading before returning an

“Unstable Scale” failure status.  Range is 5-

10 seconds.

5	Reserved			unn05	US	Reserved.

- 6		Feed Override Time		unn06	US	Setup. Time in seconds that any external

- logic (such as Slow Stem Timer or an

- operator changing modes) will be inhibited

- from removing the permissive on the

- enabling logic that is controlling the Final

- Control Element (FCE). This is used to

- prevent  something other than the Fast-Cut-

- Off from closing the FCE, which might cause

- the PAC to develop erroneous data for

- subsequent update of its constants.

- The PAC sets a status indicating that the

- Material Transfer is within the feed override

- Time. Default = 20 seconds

- 7		Minimum Slow Step Time		unn07	US	Setup.  The PAC algorithm computes a “step

- slow” value.  If the computed value is less

- than the minimum specified here,  PAC uses

- the minimum instead. Typical range is 30-60

- seconds.

- 9		Overlap Feed Alone Time		unn09	US	Setup. Time that an overlapped feed must

- fed alone before cutoff, in seconds. Typically

- 10-20 seconds.

- 10		Overlap Time Tolerance		unn10	US	Setup. Additional time in seconds allowed for

- an overlapping feed to complete.

- 11		Zero Flow Threshold		unn11	Float	Setup. Flow rate below which the system

- assumes zero flow. Typically sized 0.001%-

- 0.01% of Max Scale Capacity/second.  In a

- “dump to empty”,  the PAC uses this value to

- determine when a “dump to empty” is

- complete.  After this point, PAC keeps the

- FCE energized until the “drain time“ expires.

- When this value < 0, PAC task uses absolute

- Value for drain timer for and aborts drain

- Timer at zero flow.

- 12		Unstable Device Flow Threshold	unn12	Float	Setup. Flow rate above which PAC

- generates a “Noisy Scale” condition when

- waiting for a stable scale reading. Typically

- sized 0.005%-0.05% of Max Scale Capacity

- per second. Must be larger than Minimum

- Flow.

- 13		Minimum Add			unn13	Float	Setup.  The smallest amount of material the

- system will attempt to add. Typically 0.01%-

- 1% of Max Scale Capacity.

- 14		Maximum Unit Size		unn14	Float	Setup. Over capacity point of a vessel, in

- weight units.

- 15		Dump Trip Point			unn15	Float	Setup. Level below which PAC considers

- Dump To Empty complete. Typically,

- 0.001%-0.01% of Max Unit Size. Must be

- less than smallest batch size for the Unit.

- 16		Min PAC Flow Rate Threshold	unn16	Float	Setup.  PAC starts to apply the predictive

- algorithm when measured flow exceeds this

- value.  Typically, 0.1% of Max Unit Size.

- Must be larger than Minimum Flow.  PAC

- sets the setpoint target = SP – Spill until the

- the flow rate reaches the this value.

- 19		Reserved			unn19	Float	Reserved.

- 30		Current Zero			unn30	Float	Calculated.  After a “Dump to Empty”

- operation, PAC sets this value to actual

- scale weight when it algorithmically detects

- the zero.  It helps track a “heel” buildup in

- the vessel.

## Material-Path (i.e. Raw Material) Table

The Material-Path Table defines the routing and flow control of each Raw Material through each specific Measuring Device and Unit in the Q.iMPACT.  Each Q.iMPACT may contain the definition for all the Material-Paths for the entire Q.iMPACT cluster, but it must only necessarily have the data associated with local Material-Paths.  There may be up to 1000 Material-Paths in the Q.iMPACT cluster.

The Material-Path entry contains both static Setup parameters and dynamic PAC parameters. The System Engineer sets up the static parameters that are configuration and static processing parameters for the Material-Paths.  This data only changes when the System Engineer is “fine-tuning” the system or is reconfiguring the factory.

The Dynamic PAC parameters are variables that dynamically control the feed cutoffs.  These variables are set initially to a default value, but their values may change after every feed cycle.  Examples of these parameters are the PAC algorithm cutoff “constants.”  The PAC algorithm automatically calculates the optimum values for these variables after a minimum number of feeds. Other dynamic parameters include the counts for good and bad feeds.  Only the local Q.iMPACT has an up-to-date copy of the dynamic parameters.

ControlNet Class Code: 8C hex

An instance of the Material-Path Object may exist for each of 1000 Material-Paths.  Each contains the numbered instance attributes.

MATERIAL-PATH TABLE SHARED DATA	(External Read/Write)

Where the ‘nn’ is the instance representing the internal Unit number 1-1000, in base 36 number representation.

Attr. 	Function/Value			Name	Format	Description

1	Material-Path Assigned		qnn01	UC	Setup. 1=Yes, 0=No

2	Instrument Channel		qnn02	UC	Setup. Channel 1 to 200 is the Q.iMPACT

logical measuring device channel in the

cluster that processes this Material-Path.

3	Text Descriptor			qnn03	19C	Text Descriptor for Material-Path ID

5	PAC Algorithm			qnn05	UC	Setup.  PAC Algorithm.

0 = Spill Only,

1 = K1,

2 = K2,

3 = Dump To Empty.

6	Feeder Type			qnn06	UC	Setup.   Feeder Type

- Gain In Weight,

- Loss In Weight,

- Flow Meter

- Hand Add

7	Slow Step Timer Factor		qnn07	F	Setup.  The Slow Step Timer is defined

Factor*( target / average flow ).  The factor

is normally set as 1.5, but is adjustable on a

material basis.

If SST Factor< 0, used absolute value for

SST Factor, but generate alarm only

When SST expires.

9	Minimum Open Time		qnn09	US	Setup.  Time in seconds that spill

compensation is not applied immediately

following control device opening. If = 0, no

minimum.   It is the minimum time in

seconds that a feed must be active

before it is considered “successful” and

will have its PAC parameters updated.

This check guarantees that the flow rate

is valid before PAC updates the PAC

parameters for a Material Path.

10	Drain Time			qnn10	US	Setup.  Time in seconds that the system will

wait for material to drain into a vessel after

feed is complete and before testing for

material delivery tolerance.

11	Average Flow Rate Low Limit	qnn11	Float	Setup. Lower alarm limit for the Average

Flow Rate “A”. Typically set at 50% of the

flow rate, in weight or volume units/second.

12	Average Flow Rate High Limit	qnn12	Float	Setup. Upper alarm limit for the Average

Flow Rate “A”. Typically set at 150% of the

flow rate, in weight or volume units/second.

13	Average Spill Low Limit		qnn13	Float	Setup.  Lower alarm limit for the Average

Spill “AA”. Typically set at 50% of the

average spill, in weight or volume units.

14	Average Spill High Limit		qnn14	Float	Setup.  Upper alarm limit for the Average

Spill “AA”. Typically set at 150% of the

average spill, in weight or volume units.

15	Algorithm Update Parameter	qnn15	Float	Setup. PAC uses this value in calculation of

Average Flow Rate “A”, Average Spill “AA”,

and Cutoff Constants “B”, “BB”, and “C”.

It controls how quickly the system

responds to a change in operating

conditions. Range is 0.0–1.0. Smaller values

(0.1–0.3) should be used for systems that

have few changes, while larger values (0.6-

0.8) are used for systems that change

frequently. Default value is 0.2.

16	Flow Rate Filter Sample Period	qnn16	UC	Setup.  This value determines specifies the

period of time, from 1 to 60 seconds, over

which the rate is calculated.   For lower

values,  PAC responds more quickly to

changes in rate.  For larger values, the rate

value changes more smoothly.

17	Destination Channel		qnn17	UC	Setup. Destination channel.  Range 0 –

200. If the Destination Unit = 0, PAC

disables Unit Verification and Batch Count

Checking. At beginning of the feed, the PAC

finds the weight in the destination channel to

determine if this Material Feed will cause an overflow in the destination channel.  At the end of the feedings, the PAC determines if the loss in weight in the source channel matches the gain in weight in the destination unit.  Note that there may be multiple concurrent feeds into the destination channel. Destination Channel = 255 indicates that the Destination is outside of this cluster.

18	Max Flow Rate Alarm Value.	qnn18	Float	Setup.  Flow rates above this value generate

and alarm and terminate the feed.  Value = 0

turns off alarm checking.

19	Average Flow Rate “A”		qnn19	Float	Calculated/ Initialized in Setup. Average

Flow Rate “A”, in weight or volume units /

second.

20	Average Spill “AA”		qnn20	Float	Calculated/ Initialized in Setup. Average Spill

“AA”, in weight or 	volume units.

21	Cutoff Constant K1		qnn21	Float	Calculated.  First-order cutoff parameter

used in spill calculation.

22	Cutoff Constant K2		qnn22	Float	Calculated. Second-order cutoff parameter

used in spill calculation.

23	Flow at Last Cutoff “Q”		qnn23	Float	Calculated. Material flow rate at the cutoff of

the last feed cycle, in weight or volume units

/ second.

24	Spill at Last Cutoff		qnn24	Float	Calculated. Material spill following the last

feed cycle, in weight or volume units.

25	Cutoff Constant B			qnn25	Float	Calculated.  PAC uses “B” in K2 model. It is

the “Average Flow Squared”.

26	Cutoff Constant BB		qnn26	Float	Calculated.  PAC uses “BB” in K2 model.  It

is the “Average Flow * Spill”.

27	Cutoff Constant C			qnn27	Float	Calculated.  PAC uses “C” in K2 model.  It is

the “Average Flow Cubed”.

28	Consecutive Good Feed Counter	qnn28	Long	Calculated. Counts consecutive good feeds.

29	Total Good Feed Counter		qnn29	Long	Calculated. Counts total good feeds.

30	Total Bad Feed Counter		qnn30	Long	Calculated. Counts the total bad feeds.

Feeds aborted early in the Material Transfer

cycle are not counted as bad feeds.

31	Diagnostic Status Bits		qnn31	Long	Calculated. PAC sets feed diagnostic status

word at  completion of last feed.  A bit SET

means:

- OK to process requested feed

- Minimum Open Time passed OK

- Minimum Feed Time passed  OK

- Predictive algorithm started  OK

- Predictive algorithm completed OK

- Feed Complete & Waiting for Stable Scale

- Over Tolerance

- Under Tolerance

- Flow Rate within limits

- Flow rate within 50% of range

- Flow rate within 75% of range

- Spill within limits

- Spill within 50% of range

- Spill within 75% of range

- Primary Feed Alone

- Awaiting Start Stability

- Dump Trip Point Passed

- Validating Aggregate Feed

- Checking Fast Flow Rate Alarm

- Hand Add Complete Detected

- Feed Aborted Awaiting Drain

- Power Failure During Feed

- Configuration Reset During Feed

- Entered Setup During Feed

- ETC Error on Last Primary Feed

- – 31 Reserved

32	Last Material Transfer Status	qnn32	UC	Calculated. Last Material Transfer Status is

set at completion of feed.  Refer to arn06.