In order to receive the diagnostic data, the “Log Trace Data to LPRINT Device” must be selected as “yes” on the Ethernet web page.  Diagnostic data is sent out the default serial port and is also available on Ethernet port 1749.

All diagnostic messages now have an ID at the beginning of the message in the form of “Dnn”, where ‘D’ is an actual ‘D’ and “nn” ranges from “01” to “20” for Q.i Phase II.

Some diagnostics require that the shared data variable bx222 be set to 222 in order to be displayed.  This can be done using a simple jagBASIC program shown below:

10 defshr bx, bx222

20 bx = 222

30 end

Note: Turning off power to the Q.i or enter and exiting setup will cause bx222 to reset back to 0.

# Diagnostic ID		Method				bx222 required

D01			MATTRAN::ProcessCommand	No

Format: “D01 xxxxx mp=nnnn,ch=nnn,sq=nnn,tg=fffff,gp=nnn,ng=nnn,st=nnn vvvvvvvvvvvv”

Where:

xxxxx 	= number of quarter second ticks

mp 	= material-path

ch	= channel number

cm	= command number (see Appendix A)

sq	= sequence number

tg	= target weight (floating point)

gp	= overlapping feed group number

ng	= number of secondary overlapping feeds

st	= command status (see Appendix B)

vvv.. 	= Q.i software ID string

D02			MATTRAN::SetPrimaryTarget	Yes, set bx222 = 222

Format: “D02 mp=nnnn t=fffff i=fffff l=fffff”

Where:

mp	= material-path

t	= target weight (= initialPrimaryTarget + incrementalWeight + lastIncrement*3)

i	= incremental weight

l	= last Increment weight

D03			MATTRAN::SetPredictiveCutoffs	No

Format: “D03 HIGH FLOW nnnn aaaaaa ffffff”

Where:

nnnn	= material-path

aaaaaa	= absolute flow rate

ffffff	= average flow rate

# Diagnostic ID		Method				bx222 required

D04			MATTRAN::SetPredictiveCutoffs	Yes, set bx222 = 222

Format: “D04 mp=nnnn aaaaaa bbbbbb cccccc dddddd”

Where:

mp	= material-path

aaaaaa	= time to cutoff

bbbbbb= predicted spill

cccccc	= target weight – predicted spill

dddddd= absolute flow rate

Note: For gain-in-weight feeds only.

D05			MATTRAN::SetPredictiveCutoffs	Yes, set bx222 = 222

Format: “D05 mp=nnnn aaaaaa bbbbbb cccccc dddddd”

Where:

mp	= material-path

aaaaaa	= time to cutoff

bbbbbb= predicted spill

cccccc	= target weight – predicted spill

dddddd= absolute flow rate

Note: For loss-in-weight feeds only.

D06			MATTRAN::PreFeed)		Yes, set bx222 = 222

Format: “D06 IPT mp=nnnn t=ffffff s=ffffff”

Where:

mp	= material-path

t	= initial primary target (= command target wt. – start wt.)

s	= start weight

D07			MATTRAN::CompleteFeed		No

Format: “D07 xxxxx mp=nnnn,Dwt=fffff.ff,Dev=ffff.ff,af=ffff.ff,as=ffff.ff,s=nnn”

Where:

xxxxx	= number of quarter second ticks

mp	= material-path

Dwt	= delivered weight

Dev	= spill deviation from target

af	= average flow rate

as	= average spill

s	= last material transfer status (see Appendix C)

D08			MATTRAN::CheckAdaptiveModelBeforeFeed	No

Format: “D08 AVERAGE SPILL OUT OF BOX nnn ffffff”

Where:

nnn	= material-path

ffffff	= average spill

# Diagnostic ID		Method				bx222 required

D09			MATTRAN::ResetAdaptiveModel	No

Format: “D09 RESET ADAPTIVE MODEL nnn”

Where:

nnn	= material-path

D10			MATTRAN::ProcessSetpointTrigger	  Yes, set bx222 = 222

Format: “D10 EARLY Scale FCE off”

D11			MATTRAN::ProcessSetpointTrigger   Yes, set bx222 = 222

D12			MATTRAM::ProcessSetpointTrigger   Yes, set bx222 = 222

Format: “D12 xxxxx FC mp=nnn aaaaaa bbbbbb”

Where:

xxxxx	= number of quarter second ticks

mp	= material-path

aaaaaa	= absolute flow rate at cutoff

bbbbbb= absolute flow rate

D13			MATTRAN::UpdateWeightData	No

Format: “D13 FFA mp=nnnn aaaaaa bbbbbb cccccc”

Where:

mp	= material-path

aaaaaa	= absolute flow rate

bbbbbb= absolute flow rate at cutoff

cccccc	= max flow rate alarm limit

Note: Only displayed for scales

D14			MATTRAN::UpdateWeightData	No

Format: “D14 FFA mp=nnnn aaaaaa bbbbbb cccccc”

Where:

mp	= material-path

aaaaaa	= absolute flow rate

bbbbbb= absolute flow rate at cutoff

cccccc	= max flow rate alarm limit

Note: Only displayed for flow meters

D15			AROUTER::WaitForEvents		Yes, set bx222 = 222

Format: “D15 xxxxx RT mp=nnnn,ch=nnn,cm=nn,sq=nnn,st=nn”

Where:

xxxxx	= number of quarter second ticks

mp	= material-path

ch	= channel number

cm	= command (see Appendix A)

sq	= sequence number

st	= command status (see Appendix B)

# Diagnostic ID		Method				bx222 required

D16			UNITS::StartFeed			Yes, set bx222 = 222

Format: “D16 OVERLAP START ix=n,dch=nnn,sch=nnn,type=n,group=nnn,etc=nnn”

Where:

ix	= scale number (0 – 3)

dch	= destination channel

sch	= source channel

type	= overlap type:

1=primary feed

2=secondary overlap

3=aggregate feed

4=no overlapping feed

group	= overlap feed group number

etc	= expected time to complete

D17			UNITS::StartFeed			No

Format: “D17 VSTART ix=n,dch=nnn,sch=nnn,sec=nn,pri=nn,non=nn,ex=ffffff”

Where:

ix	= scale number (0 – 3)

dch	= destination channel

sch	= source channel

sec	= number of secondary feeds

pri	= number of primary feeds

non	= number of non-overlapping feeds

ex	= expected weight

D18			UNITS::CompleteFeed		Yes, set bx222 = 222

Format: “D18 OVERLAP DONE ix=n,dch=nnn,sch=nnn,type=n,group=nnn”

Where:

ix	= scale number (0 – 3)

dch	= destination channel

sch	= source channel

type	= overlap type:

1=primary feed

2=secondary overlap

3=aggregate feed

4=no overlapping feed

group	= overlap feed group number

# Diagnostic ID		Method				bx222 required

D19			UNITS::CompleteFeed		No

Format: “D19 VDONE ix=n,dch=nnn,sch=nnn,sec=nn,pri=n,non=nn,ex=ffffff”

Where:

ix	= scale number (0 – 3)

dch	= destination channel

sch	= source channel

sec	= number of secondary feeds

pri	= number of primary feeds

non	= number of non-overlapping feeds

ex	= expected weight

D20			MATTRAN::PrintAssembly		No

(in vessel2.cpp)

Format: “D20 mp=nnn,sss…”

Where:

mp	= material-path

sss…	= output assembly string:

“nnn,a,bbb…,fw=ffffff,wt=ffffff,rt=ffffff,ss=ccc,et=ddd”

Where:

a	= ‘*’ if data integrity bit set, else an ASCII space

nnn	= channel number

bbb…	= a series of comma separated status possibilities:

“OK” – instrument data OK

“OC” – scale over capacity

“UZ” – scale under zero

“MO” – instrument in motion

“XA” – material transfer cycle active

“FCE” – final control element on

“AK” – waiting for acknowledge complete

“Man” – in manual, not auto mode

“GW” – gross weight target

“FO” – feed override active

“FA” – feed failed

“CmE” – communications error

“StW” – device stability warning

“StX” – device very unstable

“FlH” – too high or too low flow at cutoff

“Fl3” – three times average flow alert

“FlA” – fast flow rate alarm

“OWr” – wait for all overlap requests

“OP” – primary feed in progress

“OS” – secondary overlapped feed

fw	= feed weight

wt	= gross weight

rt	= rate

ss	= slow step timer

et	= estimated time to complete

D21			MATTRAN::SetPrimaryTarget		Yes, set bx222 = 222

(in apctask.cpp)

Format: “D21 etc-dt = nnnnn, fat = nnnnn”

Where:

etc-dt	= estimated time to complete – drain time (in seconds)

fat	= feed alone time (in seconds)

D22			MATTRAN::SetPrimaryTarget		Yes, set bx222 = 222

(in apctask.cpp)

Format: “D22 Initial Flow Too Slow”

Note:	This indicates that the current dump-to-empty feed is initially 25% slower

than the previous dump-to-empty feed.

D23			MATTRAN::SetSecondaryIncrementalWeight  Yes, set bx222 = 222

(in apctask.cpp)

Format: “D23 fRC = n, chkR = sssss, qTck = sssss, abF = nnnnn, rcd = nnn”

Where:

fRC =	0 if flow rate has not yet been checked against previous flow rate

1 if flow rate check time has passed and current and

previous flow rates have been compared to see if the current

flow rate is 25% less than the previous flow rate

chkR =	time is quarter of second ticks when the flow rate will be checked

qTck =	current quarter of second ticks value

abF =	absolute flow rate

rcd =	rate check delay value in quarter of second ticks. This is set based

on the flow rate sample period (see table below)

Flow Rate Sample Period	rcd

1				12

2				20

3 or greater			36

Note: This diagnostic only applies to dump-to-empty feeds.

Appendix A – Command Codes

Code	Description

1	Start Material Transfer

2	Start Material Transfer with Gross Weight Target (scale only)

3	Start Hand Add

4	Acknowledge Material Transfer or Hand Add Complete

5	Abort Material Transfer

6	Reset Slow Step Timer

7	Start Manual Mode

8	Turn on FCE in Manual Mode

9	Turn off FCE in Manual Mode

10	Restart Auto Mode

11	Complete Feed in Manual Mode

12	Master Reset - Instrument Channel

13	Report Last Status

14	Master Reset - Cluster

15	Validate Aggregate Secondary Feeds

16	Reset ETC Error from Last Primary Feed

30	Reset Local PROFIBUS Card

31	Reset All PROFIBUS Cards in the Cluster

Appendix B – Command Statuses

## Code	Description

0	SUCCESS – Start Gain-In-Weight Material Transfer Complete

1	SUCCESS – Start Loss-In-Weight Material Transfer Complete

2	SUCCESS – Start Flow Meter Material Transfer Complete

3	SUCCESS – Start Validate Aggregate Feed Complete

4	SUCCESS – Start Hand Add Command Complete

5	SUCCESS – Command Complete

6	Command Not Complete – Request status again after a short delay

7	ERROR – Communications Error

8	ERROR – Invalid Instrument Channel Number

9	ERROR – Invalid Command

10	ERROR – Invalid Material-Path Table Index Number

11	ERROR – Invalid Algorithm in Material-Path Table Entry

12	ERROR – Invalid Feed Type in Material-Path Table Entry

13	ERROR – Invalid Measuring Device Channel Table Index in Material-Path Table Entry

14	ERROR – Invalid Gain-In-Wt Feed & Dump to Empty Algorithm Combination in Material-

Path Table Entry

15	ERROR – Invalid Destination in Material-Path Table Entry

16	ERROR – Other Invalid Data in Material-Path Table Entry

17	ERROR – Overlap Feed Request Error, Including Invalid Loss-In-Weight Feed in Material-Path

Entry and Overlapping Feed Command

18	ERROR – Invalid Data In Measuring Device Channel Table Entry

19	ERROR – Invalid Mode for Command, e.g., Controller is requesting to start a new material

transfer before the last feed is complete or before the controller has acknowledged

that the last material transfer is complete

20	ERROR – Requested add amount too small

21	ERROR – Requested add amount would bring Scale Device over capacity

22	ERROR – Scale Device Currently over Capacity

23	ERROR – Scale Device Currently under Zero

24	ERROR – Instrument Malfunction

25	ERROR – Target Weight is less than Spill

26	WARNING – Response Timeout (command still may have been processed)

27	ERROR – Too many overlapping feeds

28	WARNING – Delayed start of feed due to overlapping feed

29	WARNING – Abort ignored since Time to Complete was less than Feed Override Time

30	ERROR – Invalid overlap group number

31	WARNING – Waiting for All Secondary Requests

32	WARNING – Waiting for Measuring Device Stability

33	ERROR – Not Enough Material

34	ERROR – Device not configured or calibrated properly

35	SUCCESS – Secondary Feed Start command queued

Appendix C – Last Material Transfer Statuses

## Code	Description

0	Successful Material Transfer – K1,K2 parameters updated

1	Successful Material Transfer – Spill Only

2	Successful Material Transfer – Dump to Empty

3	Successful Hand Add

4	Material Transfer Complete – Parameters NOT updated

5	Material Transfer Complete – Parameters reset

6	Material Transfer Complete with Manual Operation

7	Failed – Unstable Scale

8	Failed – Overlapping Feed Error Corrupted Flow

9	Failed – Erratic Flow Error

10	Failed – Low Flow Error

11	Failed – High Flow Rate Alarm Error

12	Failed – Communications Error

13	Failed – Instrument Error

14	Failed – Scale Device Capacity Error

15	Failed – Predictive Algorithm Error

16	Failed – Material Transfer with Manual Operation

17	Failed – Amount of material transferred did not match in source and

destination

18	Failed – Controller Aborted Material Transfer

19	Failed – Controller Reset Channel

20	Failed – Controller Reset Cluster

21	Failed – Reserved

22	Failed – Slow Step Timer Timeout

23	Failed – Secondary Requests Timeout

24	Failed – Power Failure During Feed

25	Failed – Start Material Transfer Command Failed Immediately – Transfer

Did Not Start

26	Status Only – Material Transfer is in Progress