EDS File Installation for ControlNet. RSLogix 5, RSLogix5000 & RSLinx Recognition

Definition:EDS File.  EDS is an abbreviation for Electronic Data Sheet. It provides definitions of a device's configurable parameters and public interfaces to those parameters. Every type of configurable device has its own unique EDS. The EDS is a simple text file that allows product-specific information to be made available by a vendor for all other vendors. This makes possible user-friendly configuration tools that can be easily updated without having to constantly revise the configuration software tool.

Software:  Beginning with the following revisions, EDS (Electronic Data Sheet) files are required for RSNetworx for ControlNet, RSLinx RSLogix5 and RSLogix 5000 to recognize a device.

RSNetworx for ControlNet: Version 2.21
RSLinx; Version 1.10.176
RSLogix5: Version 4.0
RSLogix5000: Version 5.12

Symptoms:

- When a device which has no EDS file is selected OFFLINE in RSNetworks for ControlNet to be added to the Scan List, no error will be reported until you attempt to go ONLINE.  The device will display a "?" in it, unknown device.

- RSLinx can "see" a device, going online, uploading or downloading may not be possible. The EDS file for that device has not been installed

- ControlLogix modules can not be browsed in RSWho, the backplane does not show after clicking the "+" to expand the tree or the ControlLogix modules show up as a yellow "?" without a red "X".  The EDS files for the modules were not installed on the system.  This is usually the result of unchecking "ControlNet EDS files" during the install of RSLinx

Solutions:

When no EDS files have been installed, RSLinx can be reinstalled, checking          "ControlNet EDS files".

For individual files:

Getting the EDS file:

It is available on the supplied CD in the ???????/EDS_Files directory.

To installl the EDS into RSNetworx

- > Tools

- > Eds wizard

- > Register an eds file

- > Register a single file