| ## Bit |  |  |  |
| --- | --- | --- | --- |
| No. | # Hex Bit | # Dec. Bit | ### Definition |
| 0 | 1 | 1 | OK To Process Requested Feed |
| 1 | 2 | 2 | Minimum Open Time Passed OK |
| 2 | 4 | 4 | Primary Feed Delayed Until Secondary Completes1 |
| 3 | 8 | 8 | Predictive Algorithm Started OK |
| 4 | 10 | 16 | Predictive Algorithm Completed OK |
| 5 | 20 | 32 | Feed Complete & Waiting for Stable Scale |
| 6 | 40 | 64 | Over Tolerance |
| 7 | 80 | 128 | Under Tolerance |
| 8 | 100 | 256 | Flow Rate Within Limits |
| 9 | 200 | 512 | Flow Rate Within 50% of Range |
| 10 | 400 | 1024 | Flow Rate Within 75% of Range |
| 11 | 800 | 2048 | Spill Within Limits |
| 12 | 1000 | 4096 | Spill Within 50% of Range |
| 13 | 2000 | 8192 | Spill Within 75% of Range |
| 14 | 4000 | 16,384 | Primary Feed Alone |
| 15 | 8000 | 32,768 | Awaiting Start Stability |
| 16 | 10000 | 65,536 | Dump Trip Point Passed |
| 17 | 20000 | 131,072 | K1 / K2 Limit Violation2 |
| 18 | 40000 | 262,144 | Checking Fast Flow Rate Alarm |
| 19 | 80000 | 524,288 | Hand Add Complete Detected |
| 20 | 100000 | 1,048,576 | Feed Aborted Awaiting Drain |
| 21 | 200000 | 2,097,152 | Power Failure During Feed |
| 22 | 400000 | 4,194,304 | Configuration Reset During Feed |
| 23 | 800000 | 8,388,608 | Look Back Time Too Small3 |
| 24 | 1000000 | 16,777,216 | ETC Error on Last Primary Feed |
| 25 | 2000000 | 33,554,432 | Primary Feed Fed Sequentially4 |
| 26 | 4000000 | 67,108,864 | Reserved |
| 27 | 8000000 | 134,217,728 | Reserved |
| 28 | 10000000 | 268,435,456 | Available for Future Use |
| 29 | 20000000 | 536,870,912 | Available for Future Use |
| 30 | 40000000 | 1,073,741,824 | Available for Future Use |
| 31 | 80000000 | 2,147,483,648 | Substitute Spill Only Feed5 |

Notes:

- The good feeds counter of the Primary feed or any of the Secondary feeds was set to zero or less during the previous overlap feed.  This caused the Primary feed to delay starting until the Secondary feed completed.  This diagnostic bit usage replaces the unused “Minimum Feed Time Passed OK” status.

- Either the K1 factor or K2 factor (only for K2 feeds) exceeded its limit during the algorithm update at the end of the feed.  This diagnostic bit usage replaces the unused “Validating Aggregate Feeds” status.

- The Look Back Time was less than five seconds at the end of the feed.  This diagnostic bit usage replaces the unused “Entered Setup During Feed” status.

- Caused by a start weight variance violation of a secondary DTE feed.

- New bit to force the next feed to be a Spill-Only feed.