Channel Configuration defaults

For the actual description of the parameters see the Technical manual Chapter 4, subsection: assigning channels. Some parameters have recommended defaults (see below), although they will more than likely not be the most efficient settings this is a list of defaults that will get you feeding product successfully. These settings should be tuned by a specialist after initial feeds have been completed to help maximize overall system performance.

Minimum Material addition						0

Dump Trip Point							3 % of the Scale Capacity

Flow Rate Thresholds

- Zero								5 scale divisions

(NOTE: if you have a noisy process or agitator this value could be increased to 10 or 15)

- Unstable Device						2  times larger than the above mentioned Zero Threshold

- Minimum PAC						1.5  times the Zero Threshold

Skip Drain Time After Zero Flow					Leave this box unchecked

Process Times

Feed Overide Time						0

Minimum Slow Step Time					30

Stable Measuring Device Wait Time			3

Overlap Feed Alone Time					8 (Not required in Q.i Lite)

Overlap Feed Alone Tolerance				8 (Not required in Q.i Lite)