Message d’erreur

Setra Easy count

| Cnnn | Displayed when the scale is powered on. nnn equals the scale's full capacity in grams |
| --- | --- |
| busy | The scale is developing a stable reading |
| Unable | The scale is unable to complete your requested operation |
| flashing display | The weight on the pan exceeds the capacity of the scale; the pan is not properly seated or the scale is under loaded. |
| oflo | The displayed count exceeds six digits (999,999) |
| update | The scale is recalculating the average piece weight to enhance counting accuracy. |
| add nnn | The scale requests an additional number of pieces (nnn) to satisfy any minimum accuracy set up requirements. |
| sub nnn | when kitting, the scale requests pieces (nnn) be removed to satisfy any minimum accuracy set up requirements. |
| ref n.n | The scale is displaying the current software version number (n.n) |