StreeterAmet

Q-9000

# Code d’erreur

| Code | ## Description | ## Remedy |
| --- | --- | --- |
| 1 | Zero Range Exceeded | If an attempt to zero the instrument when the gross value exceeds the zero range, as entered during setup, an error 0 will occur. |
| 2 | Accumulator Overflow | This is a warning that the print format will be shifted. The error occurs when nine digits times the count increment is exceeded. |
| 3 | Saturated A/D | When an input to the A/D is over 30 millivolts or the A/D hardware fails, error 3 will occur. |
| 4 | A/D Polarity | When the input to the A/D is too negative (greater than 1.5 millivolts negative) or the A/D hardware fails, error 4 will occur. |
| 6 | Illegal Auto Tare State | When the gross weight is negative or a restriction that has been placed on the tare function during setup has been violated, error 6 will occur. |
| 7 | Tare Capacity | When the gross weight value is above the tare capacity, as entered during setup, error 7 will occur. |
| 10 | Invalid Number Entry | (In setup) when a value larger than the allowable limits is attempted, error 10 will occur. |
| 12 | Sensitivity Too Small | (In calibrate) when a test weight of insufficient size, in relation to scale capacity is used in an attempt to calibrate the scale, error 12 will occur. |
| 14 | Invalid Print Format | (In setup) when a code 11, 12, or 13 is attempted when the format table is almost filled and will be exceeded by that entry, error 14 will occur. |
| 20 | RAM Error | When the test touch switch is pressed, a check of every RAM location is made by loading a 55 reading in it, complementing it, and reading it again. If an error is detected, error 20 will occur. |
| 21 | PROM #1 Error | A check sum of PROM U16 is generated and it must equal the appropriate value, otherwise error 21 will occur. PROM U16 should be replaced and setup redone. |
| 22 | PROM #2 Error | Same as error 21 but for PROM U15 |
| 23 | PROM #3 Error | Same as error 21 but for PROM U14 |
| 60-76 | Burn Problem | Failure to burn (program the PROM) with the correct data. |
| 80-96 | PROM-Full | A test is performed just before prom burn to see if enough area is available to store the data. If the test fails, the data will not be stored and the error code is displayed. Plug a memory battery backup option into X5 to hold data in the RAM. Turn power off and replace U14 with a fresh PROM. Reapply power and burn. |