J.D. IRVING SAINT-LEANARD

-problem found

scale load cell leaning to the right, one end touching wall other end 1 inch  clearance, turned a few load  cell 45 degree to compensate for worned load cell pin which caused scale to learn, # 5 load cell damage at top and bottom and need to be replace and also others load cell have worn pin which might need to be replaced.

Also  some of the bottom receiver plates don’t seem to be align with top plate ,cells should be remove and

use alignment pins to check proper base plate alignment.