Accessing MIB

To access the MIB on a scanner do the following:

Connect directly to the scanner using the installer cable and use the AUX connection.

Set up HyperTerminal to 115200 8 N 1.

Hit the Enter key and the Space Bar several times until you see the menu come up. If after several tries you don’t get the menu, try powering down the scanner and powering it back up.

Select option 4 to erase the MIB

Reboot the scanner when complete.

To access the MIB on a SC6000 do the following:

Connect directly to the AUX port on the SC6000 with a standard serial cable.

Set up HyperTerminal to 115200 8 N 1.

Hit the Enter key and the Space Bar several times until you see the menu come up. If after several tries you don’t get the menu, try powering down the SC6000 and powering it back up.

Select option 4 to erase the MIB