Installing Firmware in “A” Scanners

ALWAYS BACK UP THE CONFIGURATION FILE BEFORE TRANSFERRING A NEW FIRMWARE VERSION

- Download firmware from FTP site

- Firmware will be a zip file

- Copy firmware to folder on your PC

- Start Genius by double clicking on icon on desktop

- Select “File”

- Select “Install Package”

- Browse to the folder on your PC where you saved the zip file

- Select the zip file and select “ok”

- Wait for Genius to Install package

- It may ask you if you want to transfer the package to the connected device. If you are connected to the scanner, then you can choose yes. If not, choose no.

- If you choose no, then connect to the device and make sure the device is powered on.

- Select “tools” and then “program download”

- Genius will connect to the device.

- Depending on the version of Genius, a window will open and you will have 2 options. The first option is to “pick from a list of device packages compatible with device model” You want to use this option.

- Click next

- Select the package you just installed

- Click next

- Allow the firmware to be transferred and verified.

- Once it has completed, click “finish”

- Genius will connect to the device and you can verify the new firmware version in the System info window that opens.

ALWAYS BACK UP THE CONFIGURATION FILE BEFORE TRANSFERRING A NEW FIRMWARE VERSION.

INSTALLING NEW FIRMWARE WILL NOT DELETE PACK TRACK VALUES.