Saving a Configuration File in Genius

- Open Genius

- Connect to the device

- Click “File” or click the “Save Current Configuration” icon 3rd from the left on the toolbar.

- Click “Save As” if you use the File option

- A window will open showing all your currently saved configuration files. These files are located in C:\Program Files\Datalogic\Genius\Files

- Type in a name under “Configuration Name” to the left of the “Save” button

- Make this name something you will recognize easily, such as “PurolatorOttawa”

- Uncheck the box just below this that reads “Include Packages”. You don’t want to save the package when you save the configuration file. This increases the file size into the megabytes.

- Click “Save”

- A “properties” window will open where you have the option of adding a description to your configuration name. You could use something like “Existing configuration before new firmware installed”

- Click “ok”

- Your configuration file is now saved.