Longacre Racing Products, Inc.

e-mail:  sales@longacreracing.com

16892 146th St. SE. Monroe, WA 98272

# Phone 360 453 2030  Fax 360 453 2031  Orders 800 423 3110

###### DX Calibration Instructions

- Set up scales where you can place an 800 lb. (363 kg) test weight on each pad.

- Turn scales on. Plug in charger. Plug cables into pads.

- Allow control box to warm up for a couple minutes and then zero all pads.

- To enter calibration mode: Press Memory buttons #3, 6, 10 at the same time.

- Gently place the test weight(s) on one scale pad. Allow the weight to stabilize for a few seconds. Press the ‘Mem Store’ button to calibrate.

- Remove the weight(s) and zero all 4 pads

- Repeat steps 5 and 6 until all 4 channels have been calibrated to their pads.

- To exit calibration mode: Press Memory #1

CHANGING KG RESOLUTION

- Get into Systest (step 4 above)

- Once in Systest press the memory#9 to toggle thru your choices

- Once you find the resolution you want press the memory#1

- Then follow the calibration instructions for your perspective load cell rating