## 1280 Checkweigher

Date : 7/28/26

Software User Guide

Scope

The 1280 Checkweigher application is included with the 1280 standard firmware and is loaded by importing the application from the Configuration menu “Import Files”.  This application is a static checkweigher with a stored low and high weight associated with an ID.  A digital output will be activated based on under, accept or over, and stores the ID, weight, status (under, accept, over), and time/date in a database.  The print key can be pressed to print the last transaction’s ID, weight, status and time/date.

Sequence of Operation

The softkeys on powerup and exit configuration are [Item] – [] – [] – [] – [Setup Menu]

Select an Item

- Operator presses the [Item] softkey.

- Indicator prompts “Enter ID to Checkweigh”.

- Operator enters a stored ID with associate low and high weights.

- The ID, Low and High weights are displayed and the systems waits for the threshold to be triggered.

Checkweigh

- When the weight exceeds the Threshold Weight the indicator checks that an ID was selected.  If not it does nothing.  If so, it waits for standstill and compares the gross or net weight (whichever mode Scale 1 is currently in), against the low and high weights:

  - Weight < Low Weight

    - Displays “UNDER”.

    - Turns on the Under output.

  - Weight > High Weight

    - Displays “OVER”.

    - Turns on the Over output.

  - Weight >= Low Weight and Weight <= High Weight

    - Displays “ACCEPT”.

    - Turns on the Accept output.

- A record will be stored to the database with the ID, the weight, the status (UNDER, ACCEPT, OVER) and time/date.

- If the auto print features is enabled (default is disabled), a ticket will print (see Serial Communications below).

- When the weight goes under the Threshold Weight the display status will clear (but the ID, Low and High values will remain) and all digital outputs will turn off.

Serial Communications – Tickets, PC

The program uses the Auxiliary Print Format #1 so that the dealer may modify the print format thru the 920i front panel or by using iRev PC software.

ID: 555
Gross:  0.25 lb
ACCEPTED

1/1/2015 01:00 PM

Application Setup & Configuration

The [Setup Menu] softkey is password-protected and offers access to the following:

- Display Program Name & Version

- Display a weight widget

| Parameter | Default | Softkey | Description |
| --- | --- | --- | --- |
| System Password | “” | Setup Password | Changing the password that is required for entry into the Setup menu. Setting the password to nothing will cause the system to not prompt for a password when the [Setup Menu] softkey is pressed. |
| Add/Edit Items |  | Database Setup -> Add/Edit Items | Allows an operator to add or edit stored IDs with associated Low and High weight values. |
| Delete Items |  | Database Setup -> Delete Items | Allows an operator to delete stored IDs that are no longer used. |
| Threshold Weight | 100 lb | Threshold Weight | This is the weight that when exceeded the system checks that an ID is selected and if so it waits for standstill before reading the weight and categorizing it based on Low and High weight. This is the same weight that when the weight goes below it clears out the last weighment information and turns off the output. |
| Auto Print Feature | Disabled | Auto Print | Allows an operator to enable/disable the auto print feature. |
| Digital IO Testing | - | More->IO Test Screen | Ability to turn off and on digital outputs. |

Database Tables

| Item Database |  |  |
| --- | --- | --- |
| Field | Type | Description |
| ID | String | Item ID |
| Name | String | Item Name |
| Low | Real | Low weight that when below the weighment is marked UNDER |
| High | Real | High weight that when above the weighment is marked OVER |

| Transaction Database |  |  |
| --- | --- | --- |
| Field | Type | Description |
| ItemID | String | Item ID |
| ItemName | String | Item Name |
| Weight | Real | Actual weight stored after standstill |
| Status | String | UNDER, ACCEPTED or OVER based on weight vs. Low and High stored weights |
| DT | DateTime | Time and date of transaction |

Hardware Setup

| Option Card Locations |  |
| --- | --- |
| Slot | Type |
| 1 | Single Channel A/D Card |

| Digital I/O |  |  |  |
| --- | --- | --- | --- |
| Slot | Bit | Type | Function |
| 0 | 1 | Output | UNDER |
| 0 | 2 | Output | ACCEPT |
| 0 | 3 | Output | OVER |
| 0 | 4 | Off | Currently Not Used |
| 0 | 5 | Off | Currently Not Used |
| 0 | 6 | Off | Currently Not Used |
| 0 | 7 | Off | Currently Not Used |
| 0 | 8 | Off | Currently Not Used |

| Serial Port |  |  |  |
| --- | --- | --- | --- |
| Port | Type | Description | Setup |
| 1 | CMD | Serial Printer | 9600,8,N,2 |
| 2 | CMD | Currently Not Used | 9600,8,N,2 |

Version Changes

| Version | Changes | Date | Firmware |
| --- | --- | --- | --- |
| 1.00 | - Initial Release | 3/1/2015 | 1.00 |
| Download: SC PF SP W DB COD SC = Standard Configuration PF = Print Formats SP = Setpoints W = Widgets DB = Database Tables COD = Program |  |  |  |