## 1280 Counting

Date : 7/31/26

Software User Guide

Scope

The 1280 Counting application is included with the 1280 standard firmware and is loaded by importing the application from the Configuration menu “Import Files”.  This application has two scales: one for sampling and one for counting.  System only supports PRIMARY units.

Sequence of Operation

The softkeys on powerup are [Sample Size] – [Scale Select] - [APW] – [] – [Setup Menu]

Enter Sample Size

- Operator presses the [Sample Size] softkey.

- System prompts – “Enter Sample Size”.

- Operator enters the size and presses the ENTER key.

- System calculates the APW based on the current weight, current scale, and current sample size.

- The number of pieces will be displayed and is constantly updated as the weight or scale selected changes.

Scale Select

- To switch between scales press the [Scale Select] softkey.

- There are two scales available.  Typically one is used for sampling (smaller capacity) and one is used for counting (larger capacity), but this application works the same for both scales, or if only one is used for everything.  The piece count is always looking at the current scale displayed.

Enter APW

- Operator presses the [APW] softkey.

- System prompts – “Enter Average Piece Weight”.

- Operator enters the APW and presses the ENTER key.

Counting Pieces

The number of pieces will be displayed and is constantly updated as the weight, scale selected, or mode selected changes.  This includes adding pieces or removing pieces as well as going to a negative net as pieces are removed from a Net Zero weight. (example:  place full box on scale, press TARE to tare off weight too net zero, and then count pieces removed).  Pressing the PRINT key will print a ticket and store a transaction.

Serial Communications – Tickets, PC

Count Ticket (Auxfmt1)

The program uses the Auxiliary Print Format #1 so that the dealer may modify the print format thru the 920i front panel or by using iRev PC software.

Gross:   10.25 lb
APW:    0.0055 lb
Pieces: 1864

1/1/2015 01:00 PM

Application Setup & Configuration

The [Setup Menu] softkey is password-protected and offers access to the following:

- Display Program Name & Version

- Display a weight widget

| Parameter | Default | Softkey | Description |
| --- | --- | --- | --- |
| System Time and Date | Current | Time/Date | Time and date of 920i |
| System Password | “” | Setup Password | Changing the password that is required for entry into the Setup menu. Setting the password to nothing will cause the system to not prompt for a password when the [Setup Menu] softkey is pressed. |
| Clear Transactions |  | Clear Trans | Allows the operator to clear the transaction database. |

Database Tables

| Transaction Database |  |  |
| --- | --- | --- |
| Field | Type | Description |
| APW | Real | Current average piece weight entered |
| Weight | Real | Actual weight at time of print |
| Pieces | Integer | Number of pieces calculated (Weight / APW) |
| TimeDate | Datetime | Time and date of transaction |

Hardware Setup

| Option Card Locations |  |
| --- | --- |
| Slot | Type |
| 1 | Dual Channel A/D Card |

| Digital I/O |  |  |  |
| --- | --- | --- | --- |
| Slot | Bit | Type | Function |
| 0 | 1-8 | Off | Currently Not Used |

| Serial Port |  |  |  |
| --- | --- | --- | --- |
| Port | Type | Description | Setup |
| 1 | CMD | Printer | 9600,8,N,2 |
| 2 | CMD | Currently Not Used | 9600,8,N,2 |

Version Changes

| Version | Changes | Date | Firmware |
| --- | --- | --- | --- |
| 1.00 | - Initial Release | 3/1/2015 | 1.00 |
| Download: SC PF SP W DB COD SC = Standard Configuration PF = Print Formats SP = Setpoints W = Widgets DB = Database Tables COD = Program |  |  |  |