System

JagBasic

TSC-365

Acadian Sea Plants ltd

For: Acadian Sea Plants ltd

By: Denis Cusson

Mettler Toledo Inc.

j 7月 aa

# System description

## Operation:

- First operator must zero the scale and select unit, if necessary.	
(Gross Weight = 00)

- Loader driver drives the empty loader onto the scale and tare the scale with remote control. Jaguar will display on his lower display the actual cumulated Net weight		(Net Weight = 00 	with loader onto the scale)

- Loader drive off the scale, load bucket with product and drive back onto the scale with full bucket.

- Driver press Print on remote control. This enter the Net Weight into Jaguar memory and print this registered Net Weight. Jaguar will also display on his lower display this registered Net Weight and flash a signal light or buzzer to warn the driver the transaction is complete.

- Loader drive off the scale, dumps load and repeat sequence #1 through #5 until loading is complete

- Wend loading is complete the operator press ‘M’ key on Jaguar terminal key pad and Jaguar will print Totalise Net Weight, display this totalise Net Weight for 10 sec and clear all accumulator.

## Display:

Main ->		Time and Date

After  Tare ->		Accumulated Net Weight	(Acc W= XXXXXX lb or kg)

After Print ->		Registered Net Weight	(NW XX = XXXXXX lb or kg)

After ‘M’->		Totalise  Net Weight	(Tot  W = XXXXXX lb or kg)

## Printing:

ACADIAN SEA PLANT LTD

Time		Date

Net Weight # XX		XXXXXXX kg or lb

Time		Date

Net Weight # XX		XXXXXXX kg or lb

Time		Date

Net Weight # XX		XXXXXXX kg or lb

Time		Date

Total Net Weight:		XXXXXXX kg or lb

See Example.

## Key Pad Activity:

### Main menu:

‘->0<-’		Zero scale.

‘->T<-’		Tare scale and switch to Tare menu.
		(act as Remote Tare)

‘S’		Toggle Unit  lb/kg	(only wend no transaction are pending)

‘M’		Print a duplicate of last Totalise Net Weight

‘ESC’		Reset all accumulator.

‘F’   ‘C’	Exit JagBasic Program and return Jaguar in scale mode.

(For technician and maintenance only)

### Tare menu:

‘<->‘		(Enter)	Register Net Weight, update accumulator, print 		registered Net Weight, clear Tare and switch to Print menu.
		(act as Remote Print)

‘C’		Clear Tare and return to main menu.

### Print menu:

‘->T<-’		Tare scale and switch to Tare menu.
		(act as Remote Tare)

‘M’		Totalise Net Weight Accumulator, Print totalised Net Weight, 		clear all accumulator and return to main menu.

‘ESC’		Return to main menu.

# System information

## Variables:

NetW$ = 	(wt102)					(6 char)

CourNW$ =	Registered Net Weight 	(last)		(6 char)

AccNW$ =	Accumulator					(6 char)

TotalNW$ =	Totalised Net Weight			(6 char)

TransNo$ =	Transaction consecutive number		(2 char)

## Port:

Printer port = Com1 in demand mode.

## Discrete:

All discrete should be set to none

Par1:

IN#1 = Remote Tare

IN#2 = Remote Print

IN#3 = Remote or Push Button Zero if need

IN#4 = Push Button Total if need.

Par2:

OUT#1 = Complete signal (Light or Buzzer )

# Plugging:

Jaguar Com 1					8856 (DB25)

TxD						  3

Gnd						  7

Jaguar Parallel Input (PAR 1)

Push Button

In1

Gnd

Jaguar Parallel Output (PAR 2)

Out1					)()()()()()(		Load

V+5

# Jaguar Setup

Com 1

9600 Baud

7 Bit

Even Parity

Flow None

Connection	Demand Scale A

Bas Terminal

Com 2

9600 Baud

7 Bit

Even Parity

RS-232

Flow None

No Connection

Parallel Input

Par1

Discrete 1	None

JagBasic

Keyboard	Kboard

Display	Jaguar

Autostart	Yes

Manual Start	Yes